# Digital Logic Practical

# Lab Report

Realization of Boolean Functions Using NAND Gate

Student:

Abhiroop Mukherjee

Teacher:

Prof. Surajeet Ghosh

Prof. Sekhar Mandal

Course:

Digital Logic Laboratory (CS351)

# Experiment 1(a)

## Objective

Realization Of AND, OR, NOR, XOR, XNOR Gate using NAND Gate

## Theory

- The AND gate is a basic digital logic gate that implements logical conjunction

  - Its Truth Table is as follows

| - A | - B | - Y = A.B |
| --- | --- | --- |
| - 0 | - 0 | - 0 |
| - 0 | - 1 | - 0 |
| - 1 | - 0 | - 0 |
| - 1 | - 1 | - 1 |

Figure 1: Symbolic Representation of AND Gate

- The OR gate is a digital logic gate that implements logical disjunction

Figure 2: Symbolic Representation of OR Gate

Its Truth Table is as follows

| - A | - B | - Y = A+B |
| --- | --- | --- |
| - 0 | - 0 | - 0 |
| - 0 | - 1 | - 1 |
| - 1 | - 0 | - 1 |
| - 1 | - 1 | - 1 |

- NOR is the result of the negation of the OR operator.

  - Its Truth Table is as follows

| - A | - B | - Y = (A+B)’ |
| --- | --- | --- |
| - 0 | - 0 | - 1 |
| - 0 | - 1 | - 0 |
| - 1 | - 0 | - 0 |
| - 1 | - 1 | - 0 |

Figure 3: Symbolic Representations of NOR Gate, Both Figure (a) and (b) are Equivalent

- XOR is a digital logic gate that gives a true (1 or HIGH) output when the number of true inputs is odd.

  - The algebraic expressions A.B' + A'.B or (A+B).(A'+B') or A⊕ B all represent the XOR gate with inputs A and B.

Figure 4: Symbolic Representation of XOR Gate

Its Truth Table is as follows

| - A | - B | - Y =A ⊕ B |
| --- | --- | --- |
| - 0 | - 0 | - 0 |
| - 0 | - 1 | - 1 |
| - 1 | - 0 | - 1 |
| - 1 | - 1 | - 0 |

- The XNOR gate (sometimes ENOR, EXNOR or NXOR and pronounced as Exclusive NOR) is a digital logic gate whose function is the logical complement of the exclusive OR (XOR) gate.

Figure 5: Symbolic Representation of XNOR Gate

Its Truth Table is as follows

| - A | - B | - Y = A XNOR B |
| --- | --- | --- |
| - 0 | - 0 | - 0 |
| - 0 | - 1 | - 1 |
| - 1 | - 0 | - 1 |
| - 1 | - 1 | - 0 |

- A NAND gate (NOT-AND) is a logic gate which produces an output which is false only if all its inputs are true. thus, its output is complementing to that of an AND gate.

Figure 6: Symbolic Representations of NAND Gate

Its Truth Table is as Follows

| - A | - B | - Y = (A.B)' |
| --- | --- | --- |
| - 0 | - 0 | - 0 |
| - 0 | - 1 | - 1 |
| - 1 | - 0 | - 1 |
| - 1 | - 1 | - 0 |

## Circuit Diagrams

Figure 9: NOR Gate using NAND Gates

Figure 8: OR Gate using NAND Gates

Figure 7: AND Gate using NAND Gates

The Following Circuits were made for the following Gates (using NAND Gate)

Figure 11: XNOR Gate using NAND Gates

Figure 10: XOR Gate using NAND Gates

Input/Output

Figure 15: Output for 1 1

Figure 14: Output for 1 0

Figure 13: Output for 0 1

Figure 12: Output for 0 0

AND Gate from NAND Gates

Figure 19: Output for 1 1

Figure 18: Output for 1 0

Figure 17: Output for 0 1

Figure 16: Output for 0 0

OR Gate using NAND Gates

Figure 23: Output for 1 1

Figure 21: Output for 0 1

Figure 20: Output for 0 0

NOR Gate using NAND Gates

Figure 22: Output for 1 0

Figure 24: Output for 0 0

Figure 27: Output for 1 1

XOR Gate using NAND Gates

Figure 26: Output for 1 0

Figure 25: Output for 0 1

Figure31: Output for   1 1

Figure 30: Output for   1 0

Figure 29: Output for  0 1

Figure 28: Output for 0 0

XNOR Gate using NAND Gates

# Experiment 1(b)

## Objective

Realization of Half Adder, Full Adder, Half Subtractor, and Full Subtractor using two input NAND Gates

## Theory

Figure 2: Realization of Half Adder

Half adder is a combinational arithmetic circuit that adds two numbers and produces a sum bit (S) and carry bit (C) as the output.

| Inputs |  | Outputs |  |
| --- | --- | --- | --- |
| x | y | C | S |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

  - From Truth Table, we observe that

    - C = x.y

    - S = x'.y + x.y' =x⊕y

- Full Adder is the adder which adds three inputs and produces two outputs. The first two inputs are A and B and the third input is an input carry as C-IN. The output carry is designated as C-OUT and the normal output is designated as S which is SUM.

| Inputs |  |  | Outputs |  |
| --- | --- | --- | --- | --- |
| A | B | C - In | Sum | C - Out |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Figure 3: Realization of Full Adder as the cascaded form of Two Half Adders

As Full Adder is used in Binary Addition, we can cascade two Half Adders to form a Full Adder

Figure 4: Realization of Half Subtractor

A half subtractor is a logical circuit that performs a subtraction operation on two binary digits. The half subtractor produces a sum and a borrow bit for the next stage.

| - Input |  | - Output |  |
| --- | --- | --- | --- |
| - x | - y | - Borrow(B) | - Difference(D) |
| - 0 | - 0 | - 0 | - 0 |
| - 0 | - 1 | - 1 | - 1 |
| - 1 | - 0 | - 0 | - 1 |
| - 1 | - 1 | - 0 | - 0 |

  - From Truth Table, we observe that

    - B = x'.y

    - D = x'.y + x.y' =x⊕y

- A full subtractor is a combinational circuit that performs subtraction of two bits, one is minuend and other is subtrahend, considering borrow of the previous adjacent lower minuend bit.

| Inputs |  |  | Outputs |  |
| --- | --- | --- | --- | --- |
| A | B | B - In | Difference (D) | B - Out |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Figure 5: Realization of Full Subtractor as the cascaded form of Two Half Subtractors

As Full Subtractor is used in Binary Subtraction, we can cascade two Half Subtractor to form a Full Adder

Figure 7: Full Adder Circuit. Here the Left Display is Sum(S) and right Display is Carry(C)

Circuits

Figure 9: Full Subtractor Circuit. Here Left Display is Difference(D) and Right Display is B-out

Figure 6: Half Adder Circuit. The Left Display is Carry(C) and the Right Display is Sum(S)

Figure 8: Half Subtractor Circuit. Here the Left Display is Borrow(B) and Right Display is Difference(D)

Outputs

- Half Adder

  - This circuit take left input bit as ‘X’ and right input bit as ‘Y’, and outputs the sum ‘X+Y’ in right output display and Carry in left output display

Figure 10: X=0, Y=0; C=0, S=0

Figure 13:X=1, Y=1; C=1, S=0

Figure 12:X=1, Y=0; C=0, S=1

Figure 11:X=0, Y=1; C=0, S=1

Figure 17: X=0, Y=1, C-in=1; S=0, C-out=1

Figure 16: X=0, Y=1, C-in=0; S=1, C-out=0

Figure 14: X=0, Y=0, C-in=0; S=0, C-out=0

Figure 15: X=0, Y=0, C-in=1; S=1, C-out=0

Full Adder

Figure 21: X=1, Y=1, C-in=1; S=1, C-out=1

Figure 20: X=1, Y=0, C-in=1; S=0, C-out=1

Figure 19: X=1, Y=0, C-in=1; S=0, C-out=1

Figure 18: X=1, Y=0, C-in=0; S=1, C-out=0

- Half Subtractor

Figure 23: X=1, Y=0; B=0, D=1

Figure 23: X=0, Y=1; B=1, D=1

Figure 22: X=0, Y=0; B=0, D=0

This Circuit give Difference as X-Y

Figure 24: X=1, Y=1; B=0, D=0

- Full Subtractor

  - This Circuit Give Difference as A - B - (B-in)

Figure 28: A=0, B=1, B-in=1; D=0, B-out=1

Figure 27: A=0, B=1, B-in=0; D=1, B-out=1

Figure 26: A=0, B=0, B-in=1; D=1, B-out=1

Figure 25: A=0, B=0, B-in=0; D=0, B-out=0

Figure 31: A=1, B=1, B-in=0; D=0, B-out=0

Figure 32: A=1, B=1, B-in=1; D=1, B-out=1

Figure 30: A=1, B=0, B-in=1; D=0, B-out=0

Figure 29: A=1, B=0, B-in=0; D=1, B-out=0

# Experiment 1(c)

## Objective

Realization of Half Adder, Full Adder, Half Subtractor, and Full Subtractor using two input NAND Gates and XOR gates

## Theory

Figure 1: Equivalent Circuit of XOR Gate made by NAND Gates

In every circuit made in Experiment 1(b), we can substitute the following arrangement of NAND Gates with XOR Gate, as they are logically equivalent.

A

B

Q

Figure 2: XOR Gate

## Circuits

Figure 3: Full Adder Circuit

Figure 3: Half Adder Circuit

Figure 5: Full Subtractor Circuit

Figure 4: Half Subtractor Circuit

## Outputs

Figure 6: X=0, Y=0; C=0, S=0

Figure 7:X=0, Y=1; C=0, S=1

Half Adder

Figure 10: X=0, Y=0, C-in=0; S=0, C-out=0

Figure 11: X=0, Y=0, C-in=1; S=1, C-out=0

Figure 9:X=1, Y=1; C=1, S=0

Figure 8:X=1, Y=0; C=0, S=1

Full Adder

Figure 15: X=1, Y=0, C-in=1; S=0, C-out=1

Figure 14: X=1, Y=0, C-in=0; S=1, C-out=0

Figure 12: X=0, Y=1, C-in=0; S=1, C-out=0

Figure 13: X=0, Y=1, C-in=1; S=0, C-out=1

Figure 16: X=1, Y=0, C-in=1; S=0, C-out=1

Figure 17: X=1, Y=1, C-in=1; S=1, C-out=1

- Half Subtractor

  - This Circuit give Difference as X-Y

Figure 18: X=0, Y=0; B=0, D=0

Figure 19: X=0, Y=1; B=1, D=1

Figure 20: X=1, Y=0; B=0, D=1

Figure 21: X=1, Y=1; B=0, D=0

- Full Subtractor

  - This Circuit Give Difference as A - B - (B-in)

Figure 23: A=0, B=0, B-in=1; D=1, B-out=1

Figure 22: A=0, B=0, B-in=0; D=0, B-out=0

Figure 25: A=0, B=1, B-in=1; D=0, B-out=1

Figure 24: A=0, B=1, B-in=0; D=1, B-out=1

Figure 27: A=1, B=0, B-in=1; D=0, B-out=0

Figure 26: A=1, B=0, B-in=0; D=1, B-out=0

Figure 31: A=1, B=1, B-in=0; D=0, B-out=0

Figure 32: A=1, B=1, B-in=1; D=1, B-out=1

# Experiment 2

## Objective

Implement BCD-to-Gray code convertor using minimum numbers of two inputs NAND gates.

## Theory

- Following Truth Table Exists for BCD to Gray Conversion

| Decimal | BCD Input |  |  |  | Gray Code Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B3 | B2 | B1 | B0 | D3 | D2 | D1 | D0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 3 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |
| 5 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 7 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 8 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |

- Using this Truth Table, we can write that

  - D3 = ∑ (8,9) + ∑d (10,11,12,13,14,15)

  - D2 = ∑ (4,5,6,7,8,9) + ∑d (10,11,12,13,14,15)

  - D1 = ∑ (2,3,4,5) + ∑d (10,11,12,13,14,15)

  - D0 = ∑ (1,2,5,6,9) + ∑d (10,11,12,13,14,15)

- Following K-Maps are made by these expression

Figure 4: K-Map for D0

Figure 2: K-Map for D2

Figure 3: K-Map for D1

Figure 1: K-Map for D3

- From these K-Maps, we find the following minterms

  - D3 = B3

  - D2 = B2 + B3

  - D1 = B2’.B1 + B2.B1’ = B2 XOR B1

  - D0 = B1’.B0 + B1.B0’ = B1 XOR B0

## Circuit

Figure 5: BCD-to-GRAY Code Converter Circuit

Based on the minterms found, following circuit was made

## Outputs

Figure 7: Output for Decimal 1

Figure 6: Output for Decimal 0

Figure 9: Output for Decimal 3

Figure 8: Output for Decimal 2

Figure 11: Output for Decimal 5

Figure 10: Output for Decimal 4

Figure 13: Output for Decimal 7

Figure 12: Output for Decimal 6

Figure 14: Output for Decimal 9

Figure 13: Output for Decimal 8